NUMBER SYSTEMS  SUPPLEMENTARY NOTES

© IT - TDT Number Systems Supplementary Notes 2Acknowledgement

• The contents of these slides have origin from School of Computing, National University of Singapore. • We greatly appreciate support from Mr. Aaron Tan Tuck Choy for kindly sharing these materials.

© IT - TDT Number Systems Supplementary Notes 3Policies for students

• These contents are only used for students PERSONALLY. • Students are NOT allowed to modify or deliver these contents to anywhere or anyone for any purpose.

© IT - TDT Number Systems Supplementary Notes 4Recording of modifications • Currently, there are no modification on these contents.

CS2100 Number Systems Supplementary Notes 5 NUMBER SYSTEMS

SUPPLEMENTARY NOTES

▪Complements

▪Floating-point Numbers

CS2100 Number Systems Supplementary Notes 6 COMPLEMENTS (1/3)

▪“Find the complement of a number” or “complement a  number” is the short way of saying… “find the negated  value in that complement system”.

▪For example, the two questions below are equivalent.

❖[4-bit] Find/get the 1’s complement of 0110

(or, 1’s complement this value: 0110)

Answer: 1001.

❖[4-bit] If x is 01101s, what is -x in 1’s complement form?  Answer: 10011s

▪So, “find the 1’s complement of 0110” is not asking for  “how is 0110 represented in 1’s complement”. See next  two slides for more examples.

CS2100 Number Systems Supplementary Notes 7 COMPLEMENTS (2/3)

▪More examples:

❖[8-bit] Find the 1’s complement of 101.

Answer: 11111010

❖[8-bit] Find the 1’s complement of 11001000.

Answer: 00110111

❖[8-bit] How is 1012 (510) represented in 1’s complement?  Answer: 000001011s

❖[8-bit] How is -1012 (-510) represented in 1’s complement?  Answer: 111110101s

❖[8-bit] Find the 2’s complement of 111000.

Answer: 11001000

❖[12-bit] Find the 2’s complement of 101.

Answer: 111111111011

CS2100 Number Systems Supplementary Notes 8 COMPLEMENTS (3/3)

▪More examples:

❖[8-bit] What is 1112 (710) in 2’s complement form?  Answer: 00000111

❖[8-bit] What is -1112 (-710) in 2’s complement form?  Answer: 11111001

❖[10-bit] What is 1410 in 1’s complement form?

Answer: (0000001110)1s

❖[10-bit] What is -1410 in 2’s complement form?  Answer: (1111110010)2s

CS2100 Number Systems Supplementary Notes 9 FLOATING-POINT NUMBERS (1/2) ▪Assume a 10-bit floating-point scheme with: 1-bit sign, 5- bit normalised mantissa, and 4-bit exponent.

▪What is this value: 1 11000 1001 ?

▪Sign bit is 1, so value is negative.

▪Mantissa is 0.110002, or 0.7510

▪What about exponent 1001?

❖If exponent is unsigned, then exponent = 9

❖If exponent is signed, and sign-and-magnitude is used, then  exponent = -1

❖If exponent is signed, and 1’s complement is used, then exponent  = -6

❖If exponent is signed, and 2’s complement is used, then exponent  = -7

CS2100 Number Systems Supplementary Notes 10 FLOATING-POINT NUMBERS (2/2)

▪Therefore,

1 11000 1001

❖If exponent is unsigned, then value is -0.112 × 29, or - 1100000002, or -38410.

❖If exponent is in sign-and-magnitude, then value is -0.112 × 2-1,  or -0.0112, or 0.37510.

❖If exponent is in 1’s complement form, then value is - 0.112 × 2-6, or -0.000000112.

❖If exponent is in 2’s complement form, then value is - 0.112 × 2-7, or -0.0000000112.

© IT - TDT Number Systems Supplementary Notes 11Q&A